n

Future Technologies Industry Cluster

College of Vocational Education

Associate Degree in Information Technology AD006

COSC 2382 Advanced  Networking 1

Case Study Answer Sheet

Semester 1 2024

This assessment task is worth 30% of your total grade

Name:  Vu Nguyen                  
Student number S4010423

Your points score for this task will be scaled to a Canvas assessment score out of 30

*You must submit this sheet and your Packet Tracer file by the due date Friday May 3, 5.30pm

*Download the Packet Tracer file from  the Assignments section on Canvas.         SECURITY CS sem 1  2024 initial.pkt

*use your eJournal, class labs and any other internet resource, including AI ( e.g. chatgpt, Bard). But you cannot discuss your work with other students. You may ask  for guidance from your teacher, preferably in classtime but also within a collaboration meeting using MS Teams or using email

*Rename this word document with you name and student number.

*Type your name at appropriate place above

Note: You only need to submit this answer sheet with your answers on Canvas.
 No need to submit the PT file

The PT file is provided for you to test your commands, if you choose

*As you complete each task enter the configuration commands in the text boxes provided for each Task in this document.

Insert just the new commands – do not copy and paste the whole running config

Sometimes, it may be easier to explain what you did, rather than paste a command

e.g. changed the subnet mask on PC11

Or, you can provide a screenshot showing what you did.

Task 1 Router Hardening   on the Hamburg router

- For failed authentication login attempts, if in a time period of 45 seconds there are 4 failed login attempts seconds , a user will have to wait for 2 minutes

- Create a local username of Admin with password Admin_2382

- This user  called Admin gains privileged mode access immediately after successful authentication.

- Only 4 simultaneous, secure vty connections are allowed for access to the  Hamburg vty lines.

- For successful remote connections on these  vty lines, disconnect the session if there is no activity for 125 seconds.

- Don't allow any PC with an ip address in the first half of the network 192.1.1.0/24  (e.g. PC2 , 192.1.1.25) to access these 4 vty lines. Use a standard ACL number 29 to configure this. Permit all other devices in the network 192.1.1.0/24  to access these 4 vty lines
To clarify, the first half are the ip addresses are from 192.1.1.0 to 192.1.1.127
Note: this type of ACL was not covered in class. Use CHATGPT or other resource to find the correct ACL

- For ssh connections, only allow version 2.

- During ssh negotiation with a client, the router will only wait 30 seconds for the client to respond with a password before considering it as a failed attempt.

Task 1  Be careful  to paste the commands under the correct  heading

All commands are on the Hamburg router

A  in a time period of 45 seconds there are 4 failed login attempts seconds , a user will have to wait for 2 minutes

login block-for 120 attempts 4 within 45

B, C   username of Admin with password Admin_2382, privileged mode

username Admin privilege 15 password Admin_2382

D,E  4 simultaneous, secure vty connections, inactivity timer 125 secs,

line vty 0 3

login local

exec-timeout 2 5

transport input ssh

F  ACL 29 on 4 vty lines; deny 1st half of 192.1.1.0/24  Use CHATGPT?

access-list 29 deny 192.1.1.0 0.0.0.127

access-list 29 permit 192.1.1.128 0.0.0.127

line vty 0 3

access-class 29 in

G,H  SSH version 2, router waits 30 seconds for ssh client to respond

ip ssh version 2

ip ssh time-out 30

Task 1  continued

- Use a secret password of cisco0808 to create a privilege level 8

- Configure a user with username cisco-08 to access level 8.

- This user is granted privilege level 8 with the password Cisco_08!

- Log messages are to be sent from Hamburg to the server at 142.1.1.253

- Time stamped log messages are seen whenever someone changes their privilege level (e.g. from user mode to privileged exec mode).

- Hamburg router will synchronize its time using the NTP service at 142.1.1.254

I  privilege level 8, with secret password cisco0808

enable secret level 8 cisco0808

J,K   user  cisco-08,  password Cisco_08!

username cisco-08 privilege 8 secret Cisco_08!

L   log messages to server

logging 142.1.1.253

M  time stamped log messages  whenever change in privilege level

service timestamps log datetime msec

N  NTP

ntp server 142.1.1.254

Task 2a  AAA  on the Berlin2 router

- Configure a AAA method that will allow users wanting console access to be authenticated by the Berlin2 AAA RADIUS server at 202.1.1.100

If the server is unavailable, use the enable password to authenticate

This AAA method will initially apply to all lines on the Berlin2 router.

- Users wanting remote access will also be authenticated by the Berlin2 AAA RADIUS server using the named method called Remote_Access.

During this testing phase, apply this named method only on the first vty line
If the server is unavailable, authentication fails

Task 2a  All commands are in the Berlin2  router

A  AAA  method for all lines, RADIUS server, backup => enable password

aaa new-model

aaa authentication login default group radius enable

radius-server host 202.1.1.100 key enable

aaa authentication login default group radius enable

line con 0

login authentication default

B  AAA named method REMOTE_Access,  RADIUS server, no backup

aaa authentication login Remote_Access group radius

line vty 0

login authentication Remote_Access

Task 2b  ipv4 ACLs on the Hamburg2  router

- On the Hamburg2 router, write a named extended ACL called TEST24.

This ACL will match the following types of traffic (match with permit statements)

a) secure web requests from PC21 (192.1.1.20) to the server at 142.1.1.253

b) ping requests from the Hamburg2 LAN (192.1.1.0/24) to any destination

c)  telnet reply packets from a telnet server on the Hamburg2 LAN with any source address going to any destination

d) ftp from any source address to the Munich2 router's fa0/0 address (142.1.1.2)

e) include an ACE (Access Control entry = a single ACL line) on line 950 that will deny all other ip traffic

Task 2b  All commands are in the Hamburg2  router

Write the command to ….create the named ACL called TEST24

ip access-list extended TEST24

a   secure web from PC21 (192.1.1.20) to the server at 142.1.1.253

10 permit tcp host 192.1.1.20 host 142.1.1.253 eq 443

b    ping requests from the Hamburg2 LAN (192.1.1.0/24) to any destination
 20 permit icmp 192.1.1.0 0.0.0.255 any echo

c  telnet reply packets from a telnet server with any source address going to any destination

30 permit tcp any any eq telnet established

d  ftp packets from any source address to the Munich2 router's fa0/0 address (142.1.1.2)

40 permit tcp any host 142.1.1.2 eq ftp

e   line 950 that will deny all other ip traffic

950 deny ip any any

Task 2c Into to VPNs

This question does not refer to any part of the Packet tracer topology

- Write  the Phase 1 commands to satisfy these requirements:
*authentication  key is adv_net1_24
*integrity uses md5
*the router’s peer address is 200.1.1.80
*Use the most secure, Diffie-Hellman setting that is available on  a Packet Tracer 1841 router (all the routers in your Case Study topology are 1841 routers)

- Write  the Phase 2  commands to satisfy these requirements:
*transform set name is TRANS-2384 and uses aes and md5
Don’t configure any crypto map settings

- Only traffic matching ALC140 may use the VPN tunnel.
This ACL 140 matches traffic that goes from the LAN 201.1.1.0/24 to the LAN 180.25.45.0/24

Task 2C   VPNs

A    Phase 1 commands

cypto isakmp key adv_net1_24 address 200.1.1.80

Hash md5

Group5

B     Phase 2 commands
crypto ipsec transform-set TRANS-2384 aes md5-hmac

Mode tunnel

C    ACL 140

access-list 140 permit ip 201.1.1.0 0.0.0.255 180.25.45.0 0.0.0.255

Task 3a Intro to the ASA
This question does not refer to any part of the Packet tracer topology

Configure an ASA for the following requirements

- Hostname ASA-Adv-Net1

- Assume that the  interface - int VLAN 1 - currently has no configuration at all.  Configure it to be an interface with name inside and with an ip address of 172.16.20.254/24

- Clients belonging to the inside VLAN 1 need to acquire ip addressing dynamically.
Use a pool that starts at 172.16.20.10 and ends at 172.16.20.22

- Configure a default route pointing to the next hop address 210.65.51.1

Task 3a  does not refer to any PT topology

A hostname

hostname ASA-Adv-Net1

B   configure the inside interface int VLAN 1

interface Vlan1

nameif inside

ip address 172.16.20.254 255.255.255.0

no shutdown

C dhcp for inside vlan 1 PCs

dhcpd address 172.16.20.10-172.16.20.22 inside

Dhcpd enable inside

D  default route

Route outside 0.0.0.0 0.0.0.0 210.65.51.1

Task 3b ZBF    on the Munich3 router

Go inside the cloud for Task 3. Find the Munich3 router

- Configure a class map to match either icmp or NTP traffic. Call  the class map ALLOW_ICMP_NTP

- Configure a policy map called Policy_Munich3  that will inspect the traffic for the class map called ALLOW_ICMP_UDP. Audit log messages should be generated when a ZBF inspection occurs.

- Create two zones called Zone_INS and Zone_OUTS

- Configure a zone pair called TRUST-UNTRUST with source zone Zone_INS and destination zone Zone_OUTS

- Apply the policy map called Policy_Munich3  to this zone pair

Task 3b  All commands are on the Munich3 router

A class map

class-map type inspect match-any ALLOW_ICMP_NTP

match protocol icmp

match protocol ntp

B policy map and log messages when inspection occurs

policy-map type inspect Policy_Munich3

class type inspect ALLOW_ICMP_NTP

inspect log

C  create zones

zone security Zone_INS

zone security Zone_OUTS

D,E create zone pair and service policy

zone-pair security TRUST-UNTRUST source Zone_INS destination Zone_OUTS

service-policy type inspect Policy_Munich3

DO NOT PLACE ANY INTERFACES INTO ZONES!!!!

Task 3c  Classic Firewall  on the Hamburg31_test router, inside the area with a yellow color

Ham31_Test_Server and PC36 are part of the trusted inside network

- On Hamburg31_test, there is already an ACL to deny all ip traffic
Write the command to apply this ACL  inbound on an appropriate interface.

- On Hamburg31_test, configure a named ACL called ALLOW_TRUSTED to permit tcp,  udp and icmp echo packets .   This ACL is already applied to an interface

- On Hamburg31_test, configure an inspection rule called FW_CHECK that inspects appropriate packets

- Apply this inspection rule inbound on the Hamburg31_test fa0/0

Task 3c All commands are on the Hamburg31_test router

A apply ACL that denies all packets

interface fa0/1

ip access-group 10 in

B ACL for trusted traffic

ip access-list extended ALLOW_TRUSTED

permit tcp any any

permit udp any any

permit icmp any any echo

C,D create an inspection and apply it

ip inspect name FW_CHECK tcp

ip inspect name FW_CHECK udp

ip inspect name FW_CHECK icmp

interface FastEthernet0/0

ip inspect FW_CHECK in

Task 4a GRE Tunnel   on the Munich3 and Frankfurt3 routers

Configure  a GRE tunnel to allow communication between PCs on Munich3's fa0/1 LAN (10.1.1.0/24)  and PCs on Frankfurt3's fa0/0 LAN (10.1.50.0/24).

The address of the tunnel is 172.16.200.12/30

- Configure tunnel0 on Munich3 and tunnel0 on Frankfurt3

- Use static routing

Task 4a  Commands are on the Munich3 router and the Frankfurt 3 router

A tunnel creation and addressing

!Munich3

interface Tunnel0

interface tunnel0

ip address 172.16.200.1 255.255.255.252

tunnel source fa0/1

tunnel destination 10.1.50.1

!Frankfurt 3

interface Tunnel0

interface tunnel0

ip address 172.16.200.2 255.255.255.252

tunnel source fa0/0

tunnel destination 10.1.1.1

B tunnel static routing

!Munich3

ip route 10.1.50.0 255.255.255.0 tunnel0

!Frankfurt 3

ip route 10.1.1.0 255.255.255.0 tunnel0

Task 4b  ipv4 host (personal) firewalls   on PC23

On PC23, 142.1.1.252, configure its personal ipv4 firewall for the following:

- allow PC23 to ssh to the Munich server 142.1.1.253

- deny udp  from the LAN 192.1.1.0/24

permit all other traffic

Task 4b   ipv4 host Firewalls

In the space below, past a screen dump of the personal firewall settings on PC23

A    Allow PC23 to ssh to the Munich server 142.1.1.253.

B     deny udp from the LAN 192.1.1.0/24.

C    permit all other traffic.