F

Future Technologies Industry Cluster

College of Vocational Education

Associate Degree in Information Technology AD006

COSC 2383 Advanced  Networking 2 
Case Study ANSWER SHEET

Semester 1 2024

This assessment task is worth 30% of your total grade

Name:  __________________                  
Student number  __________________

Instructions.

Your points score for this task will be scaled to a Canvas assessment score out of 29

*You must submit this sheet and your Packet Tracer file by the due date 
Friday May 3, 5.30pm

*Download the Packet Tracer file from  the Assignments section on Canvas.         
PT Adv Net 2 CS sem 1 24 initial.pkt

*use your eJournal, class labs and any other internet resource, including chatgpt. But you cannot discuss your work with other students. You may ask  for guidance from your teacher, preferably using email

*Rename this word document with you name and student number.

*Type your name at appropriate place above

Note: You only need to submit this answer sheet with your answers on Canvas.
 No need to submit the PT file

The PT file is provided for you to test your commands, if you choose

Task 1   Switch Port Analyzer (SPAN)  on the main topology

Write the commands. It is optional to test on your PT file

Task 1 Be careful  to paste the commands under the correct  heading

A  Local SPAN Sw2

monitor session 1 source interface FastEthernet 0/10

monitor session 1 destination interface FastEthernet 0/2

!small screenshot of sniffer settings

D     RSPAN

!Sw1, Sw2

vlan 350

remote-span

!Sw1

monitor session 2 source interface FastEthernet 0/18

monitor session 2 destination remote vlan 350

!!

!Sw2

monitor session 2 source remote vlan 350

monitor session 2 destination interface FastEthernet 0/2

Configure local SPAN on Sw2 to monitor only ping and secure web (normal and secure browsing) frames coming in or going out of the port connected to the Main_Server on port 10.

- Configure RSPAN commands on Sw1 and on Sw2 so that any frames sent out of Sw1 port 18 will use vlan 350 to be sent across the trunk link to be captured by the sniffer on Sw2 (port 2)

Task 2 Troubleshoot IVR on a router, IVR  on an MLS, EIGRP

on the main topology

Write the commands. It is optional to test on your PT file

- Vargas router has been correctly configured with subinterfaces  for VLANs 10 and 20.  However PC6 (VLAN 10) cannot ping PC8 (VLAN 20)

Troubleshoot and fix any problems => write or show what you did to fix the problem

- Configure the MLS, called DLS1, so that it has a VLAN 20 address that uses the 2nd last address in the VLAN 20 subnet. Make sure that this interface is up.

- Configure EIGRP on DLS1 so that all active interfaces are enabled for EIGRP.
Use a single eigrp network statement to achieve this.

- Modify the eigrp configuration on the Merida router so that it is an eigrp neighbor of the DLS multilayer switch.  Only enable EIGRP on the Merida link to DLS not on the other Merida interfaces

Task 2

A   Troubleshoot IVR between PC6 and PC8

interface fastethernet 0/0.10

encapsulation dot1Q 10

ip address 172.1.10.1 255.255.255.0

no shutdown

interface fastethernet 0/0.20

encapsulation dot1Q 20

ip address 172.1.20.1 255.255.255.0

no shutdown

interface fastethernet 0/0

no shutdown

B   MLS = DLS1

interface vlan 20

ip address 172.1.20.254 255.255.255.0

no shutdown

C   EIGRP on DLSw

Router eigrp 1

No au

router eigrp 1

network 172.1.0.0

no auto-summary

D EIGRP on Merida

Router eigrp 1

No au

network 172.1.10.0 255.255.255.0

passive-interface default

no passive-interface fastethernet 0/0

no auto-summary

Task 3 Etherchannel  on the main topology

- Configure a layer 3 etherchannel  group 43 between Sw3 and Sw4 for their connected ports fa0/1 and fa0/2. Use a standards based protocol.

- This layer 3 channel belongs to the network 172.1.100.48/30. Show the addressing on Sw3 only

- On Sw3 and Sw4 enable OSPF on this network 172.1.100.48/30 but don’t use any ospf network command (i.e. use an OSPF technique that does not use an ospf network statements).

Task 3  Be careful  to paste the commands under the correct  heading 
Write the commands. It is optional to test on your PT file

A    Sw3,Sw4  L3 etherchannel, gp 43, standards based

interface range fastethernet 0/1 - 2

no switchport

channel-group 43 mode active

B    ip addressing  172.1.100.48/30

!Sw3  ip addressing

interface port-channel 43

ip address 172.1.100.49 255.255.255.252

no shutdown

C    Sw3,Sw4  ospf

ip routing

router ospf 1

passive-interface default

no passive-interface port-channel 43

D    Sw3,Sw4  Load balancing

port-channel load-balance src-mac

Load balance using source mac address

Task 4 Dot1x authentication  on the main topology

Write the commands. It is optional to test on your PT file

One valid dot1x credential uses a  username of Leonardo52 and a password of
 Cisco57-6-01

- Configure Sw1 so that any user on Sw1 PC6 (port  9) needs to be authenticated using dot1x  services from the Main_Server (172.1.10.100). Assume the AAA password on the server is correct.

- Sw1 may also use a backup aaa server at 172.1.10.101 (this server is not shown on the PT file). This backup server uses the same key as the Main_Server for the client Sw1 Write what needs to be done on this backup server to be ready to authenticate dot1x clients if the Main_ Server at 172.1.10.100 fails.

- Also, configure Sw1 to use this backup server.

- Setup PC6 to successfully authenticate using dot1x

Task 4  Be careful  to paste the commands under the correct  heading

!A Sw1  commands to enable aaa and dot1x to use a main server at 172.1.10.100; there is a PC6 on  Sw1 port 9

!aaa new-model

aaa authentication dot1x default group radius

radius-server host 172.1.10.100 key Cisco57-6-01

dot1x system-auth-control

interface fastethernet 0/9

switchport mode access

dot1x port-control auto

!B In the space below, write what actions needed to be done on the Backup_server

Ensure RADIUS Service is Running

Use the Same Key: The backup server needs to use the same shared secret key.

Correct Client Configuration.

! C write the command on Sw1  for a backup server
radius-server host 172.1.10.101 key Cisco57-6-01

!D In the space below, paste a small screenshot of the dot1x settings on PC6  should be Leonardo52  and 57-6-01

Task 5 DHCP inside the cloud called DHCP

Write the commands. It is optional to test on your PT file

Task 6a ipv4  DHCP on an MLS

A    MLS_DHCP is already configured with a DHCP pool for VLAN 20 devices.

Do further configuration on MLS_DHCP so that dhcp works for this VLAN 20 pool

If you decide to configure your PT file, you may test this DHCP service on the VLAN 20 PCs

Task 6b ipv4  DHCP on a server

B    The  Internal_Server is preconfigured with a DHCP pool is for local clients. Modify the server settings so that Cisco VoIP phones may obtain the address of a server at 172.10.1.10. Wireless access points need to obtain the address of their LAN Controller at 172.10.1.20 Use a screenshot to show the new settings

Task 5a  Be careful  to paste the commands under the correct  heading

A     MLS_DHCP  Add configuration to MLS_DHCP

ip dhcp pool VLAN20

network 172.1.20.0 255.255.255.0

default-router 172.1.20.1

ip dhcp excluded-address 172.1.20.1 172.1.20.10

interface vlan 20

ip address 172.1.20.1 255.255.255.0

B     ipv4 DHCP on a server

Task 5b  Small screenshot of the Server settings => Replace the screenshot below with your screenshot

A

Task 5c ipv6  DHCP autconfig and  stateless configuration

Write the commands. It is optional to test on your PT file

- Ensure  that only autoconfig is available for  PCs14 and 15

- Configure Router_DHCP so that the PCs connected to fa0/0 on the router  will obtain ipv6 addresses using stateless dhcp

The PCs should receive the dns address of 2001::210 and the domain name of cisco2024.com (use the pool name STATELESS_DHCP)

Task 5c  Be careful  to paste the commands under the correct  heading

!Router_DHCP   only autoconfig for PCs 14, 15

ipv6 unicast-routing

interface fastethernet 0/1

ipv6 address autoconfig

no shutdown

! B Router_DHCP   configure stateless ipv6 dhcp

ipv6 dhcp pool DHCPv6_Pool

dns-server 2001:4860:4860::8888

interface fastethernet 0/0

ipv6 nd other-config-flag

ipv6 dhcp server DHCPv6_Pool

no shutdown

Task 6  HSRP inside the cloud called HSRP

MLS-hsrp and R-hsrp are in a HSRP group 20, for vlan 20 devices
R-hsrp and MLS-hsrp already have some hsrp configuration.
Complete the hsrp configuration on R-hsrp int fa0/1 so that…

- R-hsrp is the Active HSRP device

- failover time is faster by changing the hsrp timers on R-hsrp to a Hello 2 second and hold-time 6 seconds

Modify configuration on MLS-hsrp so that

- hsrp works as expected

- packets from the PCs going to the server use the serial link between R-hsrp and R2-hsrp.

Write or show with a screenshot what needs to be done to ensure that the PCs are ready to use HSRP

Task 6 HSRP   Be careful  to paste the commands under the correct  heading

! A   Complete the R-hsrp HSRP settings

int f0/1

standby 20 ip 172.16.100

standby 20 priority 150

standby 20 preempt

! B   modify timers

Int fa0/1

standby 1 timers 2 6

! C   complete hsrp on MLS-hsrp

!on MLS-hsrp

Int vlan 20

standby 20 ip 172.16.20.100

standby 20 priority 100

standby 20 preempt

! D  packets must use the serial link when R-hsrp is active

ip route 10.1.30.0 255.255.255.0 Serial 0/0

! E  pcs are ready to use HSRP

Task 7  NAT 
Task 7 NAT -  inside the cloud called NAT

Write the commands. It is optional to test on your PT file

Servers have ip addresses from 10.1.1.32/27. The PCs have ip addresses from 10.1.1.128/27

- PCs (but not servers) on the R-NAT LAN may access the internet using the public address on the R-NAT Gi0/0/1 interface.  
Use ACL 20 to achieve this.

- An Internet_user may access the company http server (10.1.1.50) using the public address 192.1.1.50

- An Internet_user may access RADIUS (tcp, port 1812) at the company  server (10.1.1.50) using the public address 192.1.1.51.

! A    Configure R_NAT
PCs have internet using the public address on R-NAT gi0/0/1

access-list 20 permit 10.1.1.128 0.0.0.31

ip nat inside source list 20 interface Gig0/0/1 overload

interface Gig0/0/0

ip nat inside

interface Gig0/0/1

ip nat outside

! B   Write commands but don’t configure any device

users on the outside can access the company server (10.1.1.50) using the public address 192.1.1.50

ip nat inside source static tcp 10.1.1.50 80 192.1.1.50 80

! C    RADIUS (tcp, port 1812) to the company  server (10.1.1.50) using the public address 192.1.1.51.

ip nat inside source static tcp 10.1.1.50 1812 192.1.1.51 1812